ROUTER CONFIGURATION GUIDE

Project: University Campus Networking System
Platform: Cisco Packet Tracer 8.2.2
Routing Protocol: RIP Version 2

1. Cloud Router Configuration

Purpose

The Cloud Router provides connectivity between the simulated external network and the university campus. It also exchanges routing information with the Main Campus Router using RIP Version 2.

Configuration Summary

Enabled Interfaces

- GigabitEthernet0/0

- Serial0/1/0

Assigned IP Addresses

| Interface | IP Address | Purpose |
| --- | --- | --- |
| GigabitEthernet0/0 | 20.0.0.1/30 | External Cloud Network |
| Serial0/1/0 | 10.10.10.6/30 | Connection to Main Campus Router |

Routing Protocol

- RIP Version 2

- Advertised Networks:

  - 20.0.0.0

  - 10.0.0.0

Verification

The router interfaces were enabled successfully, and routing updates were exchanged with the Main Campus Router.

2. Main Campus Router Configuration

Purpose

The Main Campus Router acts as the central routing device of the university network. It performs:

- Inter-VLAN Routing (Router-on-a-Stick)

- DHCP Relay

- Dynamic Routing

- Communication with the Branch Campus

- Communication with the Cloud Router

VLAN Gateway Configuration

| VLAN | Department | Gateway |
| --- | --- | --- |
| 10 | Administration | 192.168.1.1 |
| 20 | Human Resources | 192.168.2.1 |
| 30 | Finance | 192.168.3.1 |
| 40 | Business | 192.168.4.1 |
| 50 | Electronics & Communication | 192.168.5.1 |
| 60 | Academic Department | 192.168.6.1 |
| 70 | Student Department | 192.168.7.1 |
| 80 | IT Department | 192.168.8.1 |

WAN Interfaces

| Interface | IP Address | Connected To |
| --- | --- | --- |
| Serial0/1/0 | 10.10.10.5/30 | Cloud Router |
| Serial0/1/1 | 10.10.10.1/30 | Branch Campus Router |

Services

- Router-on-a-Stick

- DHCP Relay using IP Helper Address

- RIP Version 2

- VLAN Gateway Routing

Routing Networks

The router advertises:

- 10.0.0.0

- 192.168.1.0

- 192.168.2.0

- 192.168.3.0

- 192.168.4.0

- 192.168.5.0

- 192.168.6.0

- 192.168.7.0

- 192.168.8.0

Verification

Successful routing was verified using:

- show ip route

- show ip interface brief

- End-to-end ping tests between Main Campus and Branch Campus

3. Branch Campus Router Configuration

Purpose

The Branch Campus Router connects the remote campus with the Main Campus through a serial WAN link. It provides gateway functionality for the Branch Campus VLANs and exchanges routes using RIP Version 2.

VLAN Gateway Configuration

| VLAN | Department | Gateway |
| --- | --- | --- |
| 90 | Staff | 192.168.9.1 |
| 100 | Student Laboratory | 192.168.10.1 |

WAN Interface

| Interface | IP Address | Connected To |
| --- | --- | --- |
| Serial0/1/0 | 10.10.10.2/30 | Main Campus Router |

Routing Protocol

- RIP Version 2

Advertised Networks

- 10.0.0.0

- 192.168.9.0

- 192.168.10.0

Services

- Inter-VLAN Routing

- DHCP Gateway

- Dynamic Routing

Verification

Successful connectivity was confirmed through:

- show ip route

- show ip interface brief

- Branch-to-Main Campus ping tests

4. Configuration Validation

The following verification commands were used to validate the network configuration:

show running-config

show ip interface brief

show ip route

These commands were executed on all routers to verify:

- Interface status

- IP addressing

- Routing table

- RIP updates

- Connected and learned networks

5. Overall Outcome

The router configuration successfully established communication between:

- Cloud Network

- Main Campus

- Branch Campus

The implemented configuration supports:

- Dynamic routing using RIP Version 2

- Router-on-a-Stick Inter-VLAN Routing

- DHCP Relay

- Internal Server Connectivity

- End-to-End Network Communication

- DNS, HTTP, FTP and Email Services.